# Sample VSS HW Provider

The sample VSS HW provider implements a complete, working VSS HW provider.  This provider simulates a real HW provider in most respects, including the ability to create and expose snapshots of LUNs managed by the provider.  The provider relies upon a kernel-level device driver called virtual storage driver that can dynamically create virtual disk devices.  The device driver stores the virtual disk blocks in files that are located on a physical file system.  Snapshots are created by copying these image files and then creating a new virtual disk device attached to the image file copy.

Although no real HW provider would be implemented in exactly the same way, many of the details of the sample provider implementation would be common to real hardware providers.  The sample provider has a number of limitations that restrict its usefulness as anything but a sample.  Due to the simplicity of the snapshot implementation (copying the entire image file during the commit phase) the size of the virtual device is limited to a few tens of megabytes.  The virtual disk devices being created are only visible to a single machine so transportable snapshots can only be imported on the same machine as they were created.

## Directory Layout

### VssSampleProvider\

Visual Studio 7.1 project files and VSS HW sample provider source code

### Bin\

Precompiled x86 versions of provider and kernel virtual storage driver as well as installation scripts

### Platform SDK\

Copies of required libraries from the Platform SDK, included so that the provider can be compiled without installing the full Platform SDK

### VirtualStorage\

Headers and libraries that define the interface between the sample provider and the virtual storage driver

### VSS SDK\

Copies of the required VSS HW Provider SDK headers and libraries

Warning: when implementing your provider please use the headers and libraries included with the SDK.

The sample provider can only be built and installed on the x86 platform.

## Installation

The sample provider should only be installed on a test machine since the kernel driver that is included has had limited testing.  There is a possibility that the driver could crash and lose all data on the machine.

To start with, clean install W2k3 RTM (or W2k3 Sp1 once it is released) on the test machine. If you’ve installed W2k3 RTM, you must also install KB833167 or later QFEs that include KB833167.

Install the provider by running install-sampleprovider.cmd from the Bin\ directory.

This script will:

- Uninstall any existing sample provider

- Copy the sample provider and driver files to %systemdrive%\VssSampleProvider.  This step is omitted if the script is run from %systemdrive%\VssSampleProvider.

- Install the Microsoft Test Root certificate on the machine.  The virtual storage driver is signed with this certificate and having the certificate installed will prevent unwanted popups when snapshots are created.

- Install the virtual storage driver

- Create a 20Mb virtual storage drive

- Invoke the register_app.vbs script to register the provider

Use ‘vssadmin list providers’ to verify the provider is correctly installed.

Provider name: 'VSS Sample HW Provider'

Provider type: Hardware

Provider Id: {b57190af-454a-4dd0-8afd-e57facd5d9af}

Version: 1.0

Once the provider is installed, a partition should be created on the virtual disk device.  Run diskmgmt.msc to start the disk management plug-in.  From there create and format a partition on the virtual disk.  Make sure the new partition is assigned a drive letter.

From the command prompt you should be able to use the new virtual drive as a normal disk.  Files can be copied to the virtual disk, but note that the capacity is much smaller than most real disks.

Use the vsnap utility supplied in the SDK to create a snapshot of the virtual disk, check the output of the command to verify that the sample provider was used to create the snapshot by comparing the provider ID to the sample provider ID {b57190af-454a-4dd0-8afd-e57facd5d9af}.

You can create additional virtual drives. Virtual drives can be created and deleted via the ‘vstorcontrol’ command in %systemdrive%\VssSampleProvider\Bin.

Once the provider is installed and verified to be running correctly, the VSS HW provider tests can be run (see the VSS HW Provider Tests section).  All 21 variations in the tests should run and pass.

The sample provider can be uninstalled by running %systemdrive%\VssSampleProvider\uninstall-sampleprovider.cmd

## Code Organization

The sample provider is implemented as a simple COM DLL using ATL.  The registration script supplied with the SDK (register_app.vbs) is responsible for registering the DLL as a COM+ application.  COM+ provides isolation and security without additional code in the DLL.

### VssSampleProvider.cpp

This file implements VSS registration and deregistration and provides the required COM DLL entry points.

### SampleProvider.h

This file declares the CSampleProvider class that represents the actual implementation of the provider.  This class implements the interfaces required of all VSS HW provider (IVssHardwareSnapshotProvider and IVssProviderCreateSnapshotSet) and the optional notification interface (IVssProviderNotifications).

The class contains several member variables:

- m_vSnapshotInfo holds a vector that maps between the GUIDs of source and snapshot LUNs.

- m_setId and m_state are used in combination to track the state of the provider and to detect new snapshot sequences.

- The m_cs critical section protects member data, and must be acquired before accessing other member variables.

- m_vbus holds an object used to access the virtual storage driver.

### SampleProvider.cpp

This is the implementation of the CSampleProvider class and contains the bulk of the provider code.

### Utility.h

Declarations for various utility functions and classes

### Utility.cpp

Implementation of the utility functions and classes

# VSS HW Provider Tests

The VSS HW provider tests implement a basic level of testing for VSS HW providers.  The tests are straightforward functionality tests and do not provide stress, performance or long-haul coverage.  These tests should be a component of an overall testing strategy and are not sufficient on their own.

The VSS HW provider test files are located in the Tests\ subdirectory.

## Running the Tests

Before running the tests you must install a HW provider and make visible at least one volume that is managed by that provider.  That volume should be formatted and assigned a drive letter before running the tests.

Start the tests by running runtests.cmd from the Tests\ subdirectory.  This script will:

- Copy files from the Tests\ subdirectory to %systemdrive%\VssHwTests.  The script will automatically copy the correct binaries for x86 and IA64 machines.  The copying step is omitted if the script is run from %systemdrive%\VssHwTests.

- Run the tests using the default configuration file (ini\hw-any.ini)

The tests will produce output on the console to indicate progress.  At the end of the tests a summary report will be generated indicating how many variations were run and how many variations passed & failed.  A log file is produced in %systemdrive%\VssHwTests named vstestharness.log.  A successful run should result in 21 variations running and all passing.

## Test Configuration

The tests are configured via a .ini file.  For most testing the supplied hw-any.ini file in the ini\ directory should be sufficient.  This configuration will test the first HW provider in the system and will automatically detect the volumes managed by that provider.

The runtests.cmd script will use the hw-any.ini file in the ini\ directory, but it is possible to specify a different configuration file when running the tests.  Use the ‘-scenario’ command line parameter to vsstestcontroller.exe to select the .ini file to use.  For example:

vsstestcontroller -scenario={ini\myconfig.ini} -section={DEFAULT}

will run the tests with the configuration file ini\myconfig.ini.  Due to a limitation in vsstestcontroller, the configuration file must be in a subdirectory of the directory where the vsstestcontroller.exe is run from.

To create a new configuration, make a copy of the original hw-any.ini file and modify the copy.  Use the ‘-scenario’ command line parameter to specify the new configuration file.

The .ini file can be modified to specify the HW provider to be tested.  This might be useful if the system has more than one HW provider installed and the automatic detection is picking the wrong one.  Specify the provider to be used with the ‘Provider0’ key.  The key can be either ‘*’ (to select the first HW provider installed in the system) or can specify the ID of the provider to use.  For example:

Provider0={21e5ab69-9685-4664-a5b2-4ca42bddb153}

will cause the tests to only use the HW provider with the VSS ID {21e5ab69-9685-4664-a5b2-4ca42bddb153}.  Use ‘vssadmin list providers’ to get the IDs of all installed HW providers.

The volumes that should be used for testing are controlled with the ‘VolumesForProvider0’ key.  This key can either be ‘*’ (use all volumes supported by the provider) or a comma separated list of drive letters.  For example:

VolumesForProvider0=E:\,F:\

will cause the tests to only use volumes E:\ and F:\ for testing.

The configuration file can also be used to control the variations that will be run, this can be useful if you wish to omit a specific variation or only run a specific set of variations.  The ‘RegressionTestsToRun’ key controls the variations to be run.  The value of the key is a list of selections, and each selection can take one of three forms:

(1,variation)

where variation is the number of the variation to be run.  The first number in the pair represents the test number.  The only valid test number is ‘1’.

!(1,variation)

where variation is the number of the variation not to be run.  The first number represents the test number.  The only valid test number is ‘1’.

ALL

To select all variations

Examples:

RegressionTestsToRun=(1,3)

Run only variation #3

RegressionTestsToRun=ALL!(1,4)!(1,8)

Run all variations, except #4 and #8

RegressionTestsToRun=(1,8)(1,9)

Run only variations #8 and #9